# Gate valve, resilient-seated, Düker type 4004 as replacement and repair valve, Düker etec enamel

# as per EN 1171 and EN 1074 part 1-2

face-to-face length as per EN 558 basic series 15 (± 6 mm)

for potable and raw water up to 60°C

DN 80 - 200, PN 10, PN 16

Resilient-seated gate valve with adjustable and rotatable flange connections as per EN 1092-2, brand Düker.

Face-to-face length: as per EN 558 basic series 15.

Body and bonnet made of ductile cast iron material EN-GJS-500-7 (EN-JS1050)

Transition to the stem extension set as per DVGW GW 336, clamping piece and lock nut made of CuZn35NiMn2AlPb, wedge made of EN-GJS-500-7 (EN-JS1050), completely vulcanised with EPDM and vulcanised sliding shoes made of polyamide. Stem made of stainless steel, with rolled thread, material no. 1.4021, maintenance-free and medium-free corrosion-safe stem sealing through 3 O-rings and a cover cap. Connection bolts made of stainless steel A2-70, countersunk and sealed. Edge protection for bonnet and connection flange.

All parts in contact with the medium in accordance with KTW recommendation and DVGW worksheet W 270 (bacteriologically uncritical)

Actuator types (optional)

- open shaft end

- with square cap

- with hand wheel

- with stem extension set

- with electric actuator

Corrosion protection

Inside and outside shock-resistant Düker etec enamel as per the DEV (German Enamellers' Association) guidelines as well as DIN 51178 (replaces DIN 3475); resistant to class III soils (heavily aggressive soils) as per DVGW worksheet GW 9.

Models

Düker gate valve type 4004, Düker etec enamel, both sides force-locked loose flanges as replacement and repair valve

DN	PN	Qty.	Actuator type	€/pc